Task 1 – Traffic Light System: LED Circuit Calculations

# 1. Components and connections

Arduino Uno (5 V logic), breadboard, three LEDs (red, yellow, green), three series resistors (220 Ω each), and jumper wires.

Connections: D4 → red LED → 220 Ω → GND; D3 → yellow LED → 220 Ω → GND; D2 → green LED → 220 Ω → GND. Each LED must be correctly oriented (anode towards the Arduino output, cathode towards GND).

# 2. Assumed electrical values

Arduino HIGH output voltage: Vₛ = 5.0 V (nominal). Selected series resistor: R = 220 Ω per LED. Typical LED forward-voltage estimates: red Vᶠ = 2.0 V; yellow Vᶠ = 2.1 V; green Vᶠ = 2.2 V.

These forward voltages are assumptions for calculation, not measured Tinkercad values. Confirm the actual resistor settings and LED specifications in the simulation.

# 3. Current calculation (Ohm’s law)

Voltage across each resistor: Vᵣ = Vₛ − Vᶠ. LED current: I = (Vₛ − Vᶠ) / R.

| LED | Pin | Vᶠ (LED) | Vᵣ (resistor) | R | I (current) |
| --- | --- | --- | --- | --- | --- |
| Red | D4 | 2.0 V | 3.0 V | 220 Ω | 13.64 mA |
| Yellow | D3 | 2.1 V | 2.9 V | 220 Ω | 13.18 mA |
| Green | D2 | 2.2 V | 2.8 V | 220 Ω | 12.73 mA |

Worked calculations:

Red: I = (5.0 − 2.0) / 220 = 0.01364 A = 13.64 mA

Yellow: I = (5.0 − 2.1) / 220 = 0.01318 A = 13.18 mA

Green: I = (5.0 − 2.2) / 220 = 0.01273 A = 12.73 mA

# 4. Resistor selection and power rating

For an intended current of 15 mA, the resistor formula is R = (Vₛ − Vᶠ) / I. The minimum calculated values are: red 200 Ω; yellow 193 Ω; green 187 Ω. A standard 220 Ω resistor for each LED keeps the estimated current below 15 mA.

Maximum estimated resistor power (red LED): P = I²R = (0.01364 A)² × 220 Ω ≈ 0.041 W (41 mW). A standard 0.25 W resistor has sufficient power capacity under these assumptions.

# 5. Operating sequence

Red ON (3 s) → all OFF (2 s) → yellow ON (3 s) → all OFF (2 s) → green ON (3 s) → all OFF (2 s) → repeat. One complete cycle takes 15 seconds; only one LED is ON at any moment.